Robot Vision Submission Report

3R manipulator with wrist-mounted camera and fixed world-space cube

1. System Setup

Frames used in the project: {G} global frame, {B} robot base frame, {E} end-effector frame, {C} camera frame, and {I} image frame.

Fixed object used in phases 5-9: P_obj_G = [1.40, 0.60, 1.80]^T m with cube side length 0.20 m.

2. World to Image Transformation

The implemented projection pipeline is P_C = T_CG P_G = inv(T_GC) P_G, followed by s [u, v, 1]^T = K [R | t] P_G.

The pinhole intrinsics are K = [[fx, 0, cx], [0, fy, cy], [0, 0, 1]] with fx = fy = 600 px, cx = 320 px, cy = 240 px, and image size 640 x 480 px.

The camera pose is formed as T_GC = T_GB T_BE T_EC. In this project the base is aligned with the global frame, so T_GC = T_BE T_EC.

3. Image Coordinate Computation

For a given camera pose and object point, the code transforms the world point into the camera frame, checks that Z_C > 0, and computes u = fx X_C / Z_C + cx and v = fy Y_C / Z_C + cy.

Relevant outputs: outputs/phase4_image_coordinates.txt, outputs/phase5_cube_vertices.txt, and outputs/phase5_cube_image.png.

4. Joint Velocity Calculation

The geometric Jacobian column for revolute joint i is J_i = [ z_(i-1) x (o_E - o_(i-1)) ; z_(i-1) ].

The minimum-norm joint velocity is computed with q_dot = pinv(J) V. Numerical results are stored in outputs/phase6_jacobian.txt.

5. Image-Plane Trajectory of a Cube Vertex

The robot follows theta_1(t) = 30 deg + 20 deg sin(t), theta_2(t) = 45 deg + 15 deg sin(2t), and theta_3(t) = -20 deg + 10 deg cos(t).

The tracked cube vertex is (+L/2, +L/2, +L/2). The trajectory outputs are outputs/phase7_trajectory.txt, outputs/phase7_uv_plot.png, and outputs/phase7_uv_vs_time.png.

6. Simulation Video 1: 3D World View

The Phase 8 simulation shows the 3R robot motion, the magenta camera marker at the end-effector, the global frame at the origin, the wrist frame, the camera frame, the stationary red wireframe cube, and the black dashed trajectory path.

Generated files: outputs/phase8_simulation_3d.gif and outputs/phase8_snapshot_3d.png.

7. Simulation Video 2: Camera View

The Phase 9 simulation shows the 640 x 480 image plane, the principal point as a red cross, cube projections as blue dots and blue edges, and the OBJECT OUT OF VIEW message when the object leaves the field of view.

Generated files: outputs/phase9_camera_view.gif and outputs/phase9_snapshot_cam.png.

8. Verification

Run .venv/bin/python -m unittest discover -s tests and verify the generated outputs in the outputs directory before submission.

Supporting references: docs/commands_reference.md and the phase-specific docs and logs under docs/ and logs/.